# Vehicle Video Server

## Proposal

Date: 31 August 2022

Co-Confidential

Attn: Roger Yeo / Adrian / Sandy		From: Ma Zai Hang

Neo Kian Hong / Lin bing

To:    STELS		 			Fax:	64649500

Fax:	 	 					Tel:	67507007

Tel:	 						Total:  17 pages

(Inclusive of this page)

- Objective

This document is a business proposal to design / develop / fabricate the Video Server for Terrex-Gen5 Vision System, according to the technical specification requirements.

- Work scope & Responsibility

TEAMONE

- To work as a team together with Customer for video server and iDDP in system architecture design.

- To provide electronic hardware / mechanical / thermal design and related software development. (Refer to technical proposal in annex 1)

- To design and fabricate the related test equipment and develop test software for Video Server hardware performance / acceptance test.

- Develop or supply the toolkits / application software for secondary development to user’s request.

- To fabricate a minimum of 1 working prototypes (or quantity as required)

- To assist customer to integrate video / control functions / software into the video server.

- To work with customer for vehicle integration and test.

- To provide technical support during operation of ARSS Scenario Generation and Performance Validation.

- To provide technical support for EQT, EMI/EMC test.

- To provide 1-year hardware quality warranty service.

- To provide fabrication service for video server production, if any manufacturing contract is awarded to the customer/TeamOne Team.

STELS (Customer)

- System level specification, including the hardware requirements of the features for video processing function.

- To develop the software on video server to video processing function, excluding interface driver / 360 degree field of stitched view / video CODECs functions.

- To provide instructions of operating procedure & performances.

- To conduct the safety / reliability analysis.

- To conduct EQT, EMI/EMC, including booking the test facilities for the test.

- To conduct system integration / test in vehicle.

- To conduct ARSS Scenario Generation and Performance Validation.

- Schedule and milestone

- PDR: in 8 weeks upon project start, system design completed.

- CDR: in 28 weeks upon project start, initial design ready for review.

- Prototype completion in 9 months from the date of PO reception.

Annex 1.	Technical Proposal for Video Server box.

Figure 1.    Terrex-Gen5 Video Server & iDDP ---- (with built-in Network switch & embedded GPU)

Annex 2. 	Video Server proposed functionalities and performances

| Item | Description | Characteristics | Remark |
| --- | --- | --- | --- |
| 1 | General Requirements |  |  |
| 1.1 | Multiple SDI/H.264(H.265) input and output | In rugged enclosure |  |
| 1.2 | Basic functions | Supports: Video distribution in HD-SDI Networking Video stream ATD/R/T Video Stitching Video Augmentation Video Enhancement Video CODEC Video Fusion Video Embedded META data |  |
| 1.3 | Support additional GPU card, RTX3000 or equivalent | Support up to Jetson AGX Xavier module |  |
| 1.4 | Responding to UIT/iDDP | for video source changing, view selection, without affecting each other |  |
| 1.5 | Video screen image / info. Contents | including: PIP Stitched view Day/night view Sight slewing ATR augmentation Dynamic OSD |  |
| 1.6 | Interlaced and non-HD video input | performing de-interlacing and size scaling function |  |
| 1.7 | Output video backup | provide redundant video feeds for the crew |  |
| 1.8 | Output screen when no respective video feed | Raster blue screen |  |
| 1.9 | Output screen when no respective video feed | Raster blue screen |  |
| 1.10 | BIT | System , I/O , Networking |  |
| 1.11 | GPU maximum loading | <70% |  |
| 1.12 | CPU reserved capacity | >50% |  |
| 1.13 | Documentation | To provide design / specification of interface / CPU / GPU / FPGA. 1. CPU/FPGA/GPU: Xilinx UltraScale SoC 2. GPU/CPU: Nvidia Jetson AGX Xavier module |  |
| 2 | Video Interface |  |  |
| 2.1 | Support input camera interface | ARSS / Stitched view BEV Stereo Vision Driving | Through RS422, RS232, Etherent |
| 2.2 | Number of input channels | Up to 20x HD-SDI or others Support up to 16x H.264 video stream playback (from network or storage) 1x Audio | HD/SD-SDI configurable |
| 2.3 | Number of output channels | 8x HD-SDI with redundancy of H.265 stream over IP 1x optional when a GPU is added |  |
| 2.4 | Direct HD-SDI feed | 3x feed for iDDP (w/ spare channels) 5x feed for UIT optional 1x feed when a GPU is added |  |
| 2.5 | Glass-to-glass latency | HBM >16000V. |  |
| 2.6 | Independency of video feed | all video feed shall be able to operate independently, without affecting each other |  |
| 2.7 | Number of auxiliary feed | 7x stream over IP, playback enable | H.264/265 |
| 2.8 | Latency of ARSS view with ATD/R/T | <300ms | to be confirmed together with STELS software team |
| 2.9 | To propose video format to iDDP and UIT | to meet the criteria of latency | Primary video: HD-SDI auxiliary feed: H/265 over IP |
| 2.10 | Estimated video latency | <140ms for UIT & iDDP single view <200ms for stitched view in HD-SDI <300ms for auxiliary feed in H.265 format | iDDP latency would be better, to be less than 90ms. (if iDDP is from Teamone provided camera delay is <40ms) |
| 2.11 | Actual measurement of system | upon availability of hardware |  |
| 2.12 | Ethernet Switch built-in | 1x 10G Ethernet + 8x GbE to external |  |
| 2.13 | Engineering tools from Network | TBD | Remote diagnosis / configuration / operation |
| 2.14 | Communication / control interface | Ethernet through Ethernet switch 2x CAN (MIL-CAN) 4x RS422 4x RS232 |  |
| 2.15 | CAN terminal | No 120 Ohm termination resistor should be put |  |
| 2.16 | Supported input video format | HD-SDI -- SMPTE292 H.264/265 over IP |  |
| 2.17 | Supported output video format | HD-SDI -- SMPTE292 H.264/265 over IP |  |
| 3 | Discrete I/Os |  |  |
| 3.1 | Discrete 24v/0v input from vehicle | 8x ch input, 4x ch output |  |
| 3.2 |  | Opto-isolator input / OC output |  |
| 3.3 |  | Ref. with vehicle power return |  |

| 4 | Power supply |  |  |
| --- | --- | --- | --- |
| 4.1 4.2 | Operating power voltage | Nominal: 28VDC Low: 18VDC High: 32VDC MIL-STD-1275B Power consumption: Nominal: 250W continuous Peak: 450W Maximum: 350W continuous | Based on estimation of 60%, 100% , 75% loading for nominal, peak and maximum consumption |
| 5 | System clock |  |  |
| 5.1 | Timing synchronization function | Input: 1PPS from navigation system output for: video stitch and other functions |  |
| 5.2 | 1ppS re-distribution | to platform cameras |  |
| 5.3 | NTP connection | RFC5905 NTP protocol for time sync. | Through Ethernet |
| 5.4 | time stamping and video telemetry | MISB0605.6 for uncompressed video MISB0604.3 for compressed video |  |
| 6 | A/V recorder / Player ---- Optional |  |  |
| 6.1 | Recorder | for UIT screens for all camera input video |  |
| 6.2 | Recording method | Loop recording with time stamping information |  |
| 6.3 | Audio recording | sync. With timing | MPEG |
| 6.4 |  |  |  |
| 6.4.1 | Input | HD-SDI or others (TBD) |  |
| 6.4.2 | Data extraction | Secure way | USB or Ethernet. |
| 6.4.3 | Playback function | From VSR, x2/x4/x8 speed, playback portal |  |
| 6.4.4 | Trigger / Arm signal input | 24V/0V input from DDS information package. OSD on recorded video for indication of above signals |  |
| 6.4.5 | ARM signal | Start recording on VC/Gunner/Centre UIT when switched on. Stop if it is released. |  |
| 6.4.6 | Trigger signal | Start recording on VC/Gunner/Centre UIT , 15 second pre- and 30 sec post- when switched on. Stop if it is released after 30 seconds. |  |
| 6.5 | Storage media |  |  |
| 6.5.1 | Recorded video for platform camera input | Number of: 24x ch Total media volumn size: 2TBytes | TBD |
| 6.5.2 |  |  |  |
| 6.5.3 | Functional check for preview | for each video claps |  |
| 7 | DDS connectivity ---- Client (Optional) |  |  |
| 7.1 | OMG DDS security | Protocol (RTI Connext DDS Secure 5.3.1 and RTI Connext DDS Secure plugins |  |
| 7.2 | Video distribution protocol | RTP for H.265 over IP | RTP |
| 8 | Video Control and display functions |  |  |
| 8.1 | ARSS and stitched view |  |  |
| 8.1.1 | Involved cameras | respective view of day and all ti cameras on platform |  |
| 8.1.2 | Output video | Available on UITs upon request Optionally available on iDDP upon request up to 180degree / section reverse / normal view |  |
| 8.1.3 | Video truncation | to 3x 1920x1080@30Hz UIT output |  |
| 8.1.4 | Bearing information maintaining | Maintain the information in original input video |  |
| 8.1.5 | BMS function | Touch to slew from BMS |  |
| 8.1.6 | Video cross section | able to cross section stitched view | To be done in AGX Xavier module? |
| 8.1.7 | FOV display | FOV selection from screen |  |
| 8.1.8 | Zooming of AOI | AOI zoom in |  |
| 8.2 | Potential growth and tools (with additional GPU card and software from STELS) |  |  |
| 8.2.1 | Real-time ATD/ R/ T augmentations | on Stitched video on individual video streams based on user selection and from other systems |  |
| 8.2.2 |  |  |  |
| 8.2.3 | Augmentation data | Pushed from DDS For varies of type of graphics, text |  |
| 8.2.4 | Calibration tools / features | Available for operators to calibrate upon vehicle configuration | Reference table for correction based on calibration |
| 8.2.5 | Tools | Available for engineering |  |
| 8.3 | Video Image enhancement |  |  |
| 8.3.1 | Features | Sensor Fusion Contrast adjustment Brightness manipulation Warp correction | for all channels |
| 8.3.2 | Blending view | For stitched view |  |
| 8.4 | Target Detection / recognition functions (Growth Potential) |  |  |
| 8.4.1 | ATD/R function | On video input from EO cameras on video input from TI |  |
| 8.4.2 | ATD/R function vehicle speed range | 0~32KMph for stationary / moving human/vehicle objects | Figure 2: Target/ Object Classes Tree |
| 8.4.3 | ATD/R | object classification / confidence level based on training |  |
| 8.4.4 | Machine learning | Human / vehicle |  |
| 8.4.5 | target recognition | Level 3/4 |  |
| 8.4.6 | Detection | > level 2 |  |
| 8.4.7 | Improve progressively | During trail |  |
| 8.4.8 | Deep learning / Machine vision | For development |  |
| 8.4.9 | ATD/R performance | Table 3: ATD/ R at 90% Probability of Detection (PD) False Alarm Rate (FAR) <20% Miss detection (false negative) rate <10% |  |
| 8.4.10 | ATT features | As described |  |
| 8.4.11 | Result output in META data | As described |  |
| 8.4.12 | Processing time for ATD/R/T and overlay of information on OSD | <50ms | Quite difiicult??? |
| 8.5 | Video on-demand selection and connectivity |  |  |
| 8.5.1 | View | For surroundings |  |
| 8.5.2 | Command | From DDS for layout information |  |
| 8.5.3 | Video over IP through DDS | As requried |  |
| 8.5.4 | Layout size | 1920x1080 @ 30Hz Multiple sub-images |  |
| 8.5.5 | Created layout properties | Video Source ID Start co-ordination of the source image Start co-ordination of image in layout Sub-image width and height |  |
| 8.5.6 | Health status of BIT | Pushed to DDS |  |
| 8.9 | Respond delay to a command from DDS | <TBD |  |
| 8.9.4 | current layout enumeration | TO DDS |  |
| 8.9.5 | Number of sub-images in one layout | >=16 video source (can be repeated on same source) |  |
| 8.5.10 | Video latency difference | <1 or 2 frame between video sources |  |
| 8.5.11 | Not occupied area in layout | Fill in black color |  |
| 8.5.12 | Error to create video layout | fill in the error area with black color |  |
| 8.6 | OSD and ARSS view |  |  |
| 8.6.1 | OSD upload | From UIT |  |
| 8.6.2 | Tools for OSD editing, generation | Toolkit |  |
| 8.6.3 | Color of OSD | 256x color |  |
| 8.6.4 | resolution of OSD | 1920x 1080 1024x 768……. |  |
| 8.6.5 | OSD transparency | Alpha channel available |  |
| 8.6.6 | Retrieval of OSD | Available |  |
| 8.6.7 | Upload/removal of OSD | Shall not affect system stability |  |
| 8.6.8 | OSD display | Resizable upon display size |  |
| 8.6.9 | Protocol for OSD uploading | Shall be provided to customer |  |
| 8.6.10 | Development toolkit for OSD | to be developed |  |
| 8.6.11 | ARSS requirement | TBD |  |
| 8.6.12 | Proposal for ARSS | Shall be provided to customer |  |
| 8.7 | Documentation and Others (based on mass production order) |  |  |
| 8.7.1 | ATP | 1month notice |  |
| 8.7.2 | Technical support | For prototype For first 2 articles for subsequent product units |  |
| 8.7.3 | Technical support | For integration of system to vehicle |  |
| 8.7.4 | Test plan | >1 month prior to test |  |
| 8.7.5 | Test result | <1 month after test |  |
| 8.8 | Tools and trainings |  |  |
| 8.8.1 | Engineering development toolkit | Available |  |
| 8.8.2 | Training of engineering development | Available |  |
| 8.8.3 | Training contents | As required in paragraph |  |
| 8.8.4 | Provisions | Available |  |
| 8.8.5 | Hardware package for training | Available |  |
| 8.8.6 | Software package for training | Available |  |
| 9 | Qualification standards reference |  |  |
| 9.1 | Applicable documents | MIL-STD-461E MIL-STD-464C MIL-STD-1275B MIL-STD-188-124B IEC-61000-4-2 |  |
| 9.2 | EMI / EMC Requirement | CE 102 RE102 CS101/114/115/116 RE103 |  |
| 9.2.1 | Standard for qualification | MIL-STD-461E |  |
| 9.2.2 | Waiver of test item | To be approved by customer |  |
| 9.2.13 | EMI/EMC test report | a. EMC Control Plan b. EMC Test Plan/Procedure c. EMC Test Report |  |
| 9.3 | Power requirement | MIL-STD-1275B Test Plan Test report | report within 1 month after test |
| 9.4 | Environmental Qualifications |  |  |
| 9.4.1 | Applicable documents | MIL-STD-810E IEC 60529 STANAG 2895 |  |
| 9.4.2 | Test requirements | Table 5/6/7/8/9 |  |
| 9.5 | Reliability / safety & workmanship |  |  |
| 9.5.1 | Applicable Documents | MIL-STD-882E AMCP 706-125 MIL-HDBK-454B |  |
| 9.5.2 | Excessive Temperature |  |  |
| 9.5.2.1 | Wiring |  |  |
| 9.5.2.2. | Mechanical Hazard |  |  |
| 9.5.3 | System Safety Requirements for Software | Video Server and IDDP is identified to have Software Criticality Index 1 (SwCI 1) for the following hazards: a. Driver/ Commander misled by wrong camera video display b. Loss of video display during closed hatch driving |  |
| 9.5.4 | Safety Critical Item (SCI) and Reliability Critical Items (RCI) | Video Server, IDDP, BEV Cameras and Platform Cameras (Driving Camera) are classified as Reliability Critical Item |  |
| 10 | Acceptance test |  |  |
| 10.1 | Contract Data Requirements List (CDRL) | TBD |  |
| 10.2 | Integrated Logistics Support (ILS) Requirements | TBD |  |
| 10.3 | Reliability & Maintainability Requirements | TBD |  |
| 10.4 | Quality Assurance | TBD |  |
| 10.5 | Software Development Requirements | TBD |  |
| 10.6 | ARSS Requirements | TBD |  |
| 10.7 | Deliverables and Schedule | TBD |  |

Annex 3.		System Architecture

Figure 1.	Video server internal functional block diagram

Figure 3.	Video processing cell block diagram (as per in slot #1~3 in figure 2)

Figure 4.	Networking switch diagram in video server.